Open Arduino IDE and Connect Arduino to computer.

Open File > Examples > 11. ArduinoISP > ArduinoISP

Choose Board : Arduino Uno and Port COM

Upload to Arduino Uno

After upload done

Change Board : Arduino Mega 2560 and Processor : ATmega2560

We use pin 10,11,12,13 (SS,MOSI,MISO,CLK) on Arduino Uno

And connect to pins (ISP Port) on the Sensor Node

Connect pin on Arduino Uno to Sensor Node

MOSI ( pin 11 ) > MOSI

MISO ( pin12 ) > MISO

SCK ( pin 13 ) > SCK

SS ( pin 10 ) > RESET

VCC ( 5V ) > VCC

GND > GND

Choose Board : Arduino Mega 2560 and Processor : ATmega2560

Connect Arduino to computer

Choose Programmer : “Arduino as ISP” and Port COM

Then click Burn Bootloader for ATmega2560 on Sensor Node.

Wait about 30 seconds to finish

Then, Open Aruidno IDE and open (code).ino file (updated code).

You must remove ATMega328P on Arduino UNO,

be careful, foot of Atmega328 can be broken.

We use the this Arduino Board without Atmega328P

Connect UART pins to SensorNODE. Look 3 pins below the ISP Port.

TX > RX

RX > TX

GND > GND

RESET > RESET (on ISP Port)

5V > 5V (on ISP Port)

Then Upload code to ATMega2560 on SensorNode